# Network Diagram

# 2.1 Address Planning

# IPv6-Address Table

| Network-Name | IPv6 Subnet Address |
| --- | --- |
| Company HQ | AB12:4216:1000::0/64 |
| Branch A | AB12:4216:6000::0/64 |
| Branch B | AB12:4216:4000::0/64 |

# IPv4 Address and IPv6 Address Table

| Sr# | Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| 1 | mR201 | Se0/1/0 | 150.50.16.2 | 255.255.255.252 | N/A |
|  |  | Fa0/0 | 150.50.16.65 | 255.255.255.192 | N/A |
| 2 | Server0 | Fa0 | 150.50.16.67 | 255.255.255.192 | 150.50.16.65 |
| 3 | PC1 | Fa0 | 150.50.16.66 | 255.255.255.192 | 150.50.16.65 |
| 4 | mR202 | Se0/0/1 | 150.50.16.1 | 255.255.255.252 | N/A |
|  |  | Se0/0/0 | AB12:4216:2000::1/64 | /64 | N/A |
|  |  | Se0/1/0 | AB12:4216:3000::1/64 | /64 | N/A |
|  |  | Fa0/0 | AB12:4216:1000::1/64 | /64 | N/A |
| 5 | Server1 |  | AB12:4216:1000::3/64 | /64 | FE80::1 |
| 6 | PC2 |  | AB12:4216:1000::2/64 | /64 | FE80::1 |
| 7 | mR203 | Se0/0/1 | AB12:4216:5000::1/64 | /64 | N/A |
|  |  | Se0/0/0 | AB12:4216:3000::2/64 | /64 | N/A |
|  |  | Fa0 | AB12:4216:4000::1/64 | /64 | N/A |
| 8 | Server2 | Fa0 | AB12:4216:4000::3/64 | /64 | FE80::1 |
| 9 | PC3 | Fa0 | AB12:4216:4000::2/64 | /64 | FE80::1 |
| 10 | mR204 | Se0/0/1 | AB12:4216:5000::2/64 | /64 | N/A |
|  |  | Se0/0/0 | AB12:4216:2000::2/64 | /64 | N/A |
|  |  | Fa0/0 | AB12:4216:6000::1/64 | /64 | N/A |
| 11 | Server3 | Fa0 | AB12:4216:6000::3/64 | /64 | FE80::1 |
| 12 | PC4 | Fa0 | AB12:4216:6000::2/64 | /64 | FE80::1 |

# 2.2 IPv4 Routing Configuration

mR201

router eigrp 10

network 150.50.16.0 0.0.0.3

network 150.50.16.64 0.0.0.63

mR202

router eigrp 10

network 150.50.16.0 0.0.0.3

# 2.3 IPv6 Routing Configuration

mR202

ipv6 unicast-routing

interface FastEthernet0/0

ipv6 address FE80::1 link-local

ipv6 address AB12:4216:1000::1/64

ipv6 eigrp 10

interface Serial0/0/0

ipv6 address AB12:4216:2000::1/64

ipv6 eigrp 10

interface Serial0/1/0

ipv6 address AB12:4216:3000::1/64

ipv6 eigrp 10

clock rate 2000000

router eigrp 10

network 150.50.16.0 0.0.0.3

ipv6 router eigrp 10

eigrp router-id 2.2.2.2

mR203

ipv6 unicast-routing

interface FastEthernet0/0

ipv6 address FE80::1 link-local

ipv6 address AB12:4216:4000::1/64

ipv6 eigrp 10

interface Serial0/0/0

ipv6 address AB12:4216:3000::2/64

ipv6 eigrp 10

interface Serial0/0/1

ipv6 address AB12:4216:5000::1/64

ipv6 eigrp 10

ipv6 router eigrp 10

eigrp router-id 3.3.3.3

mR204

ipv6 unicast-routing

interface FastEthernet0/0

ipv6 address FE80::1 link-local

ipv6 address AB12:4216:6000::1/64

ipv6 eigrp 10

interface Serial0/0/0

ipv6 address AB12:4216:2000::2/64

ipv6 eigrp 10

interface Serial0/0/1

ipv6 address AB12:4216:5000::2/64

ipv6 eigrp 10

ipv6 router eigrp 10

eigrp router-id 4.4.4.4

# 2.4 IPv4IPv6 Network NAT-PT

ipv6 nat v4v6 source 150.50.13.100 AB12:4216:1000::3

ipv6 nat v6v4 source AB12:4216:8000::1 150.50.16.1

interface Loopback0

ipv6 address AB12:4216:8000::1/64

# 2.5 Verification

PINGING Router Mr203 with Branch B PC